ATMO/ECE 489/589 Spring 2017

Homework #7  due Thu., Apr. 20

1.	(10 pts)The sketch below illustrates some of the paths that signals from a distant lightning 	return stroke could follow between the source and a receiving antenna located 100s 	(1000s) of kilometers away.  Radiation can propagate directly to the antenna along the 	ground (this is the "ground wave").  Radiation initially directed upward can also arrive at 	the antenna after undergoing one or more reflections between the ground and a 	conducting layer in the lower ionosphere.

An actual recording of a distant return stroke electric field waveform is shown below.  	This signal was recorded at Gainesville, Florida, and was produced by a return stroke that 	occurred approximately 900 km to the ENE.  The ground wave signal as well as the first 	and second "sky wave" signals (from the 1- and 3-reflection paths in the sketch above) 	are identified.  Use the differences in the times of arrival between the ground wave and 	the 1st sky wave and between the ground wave and the 2nd sky waves to make two 	estimates of, H, the effective height of the reflecting layer in the ionosphere (you may 	assume that the signal source is at ground level, that the ground is flat, and that the radio 	waves propagate at the speed of light).

2.	(10 pts) Intersections of bearing vectors from a network of magnetic direction finding 	sensors has produced multiple lightning strike locations, (Xi, Yi).  What would the  	optimal location,(X,Y), be in a case like this?

Often in problems of this kind we try to find a solution that minimizes R2, the sum of the 	squares of the differences between the optimal location and the measured locations:

We can differentiate R2 with respect to X and Y and set each equation equal to zero to find the 	minimum:

3. 	(15 pts) The definition of a hyperbola is "the set of points in a plane whose distance to fixed 	points in the plane have a constant difference."  The object of this problem is to demonstrate the 	validity of that statement.

	Two fixed points 1 & 2 are located at (-c, 0) and (+c, 0).  A hyperbola crosses the x-axis at x = a.  	Point (x, y) is an arbitrary point on the hyperbola and is a distance d1 from Point 1 and a distance 	d2 from Point 2.  The values of  d1  and d2 will change depending on x and y but the difference d1- 	d2 will remain the same for any point on the hyperbola.

The problem has three parts:
	(a)  Show that d1- d2= 2a  (hint: since d1- d2  remains constant, you can choose any point on the 	hyperbola to determine d1- d2 )
	(b)  Demonstrate that, after some manipulation, the expression above (i.e d1 - d2 = 2a)  leads to

(c)  Show that at large values of x and y the hyperbola becomes a straight line