ATMO/ECE 489/589 Spring 2009

Homework #1 pt. 2

due Thu., Feb. 5

3a.	A point charge, Q, is located at an altitude, H, above a grounded conducting plane of 	infinite extent.

(a) Is there an attractive or repulsive force between Q and the conducting plane?

3b.	Derive an expression for:

(a) the electric field at the ground as a function of horizontal distance, D (see sketch. 		     Assume the plane is flat, it's not drawn very well).

(b) determine the surface charge density as a function of D, σ(D).

(c) integrate the surface charge density over the entire conducting plane.